FPGA-Based Smart Parking Management System with Real-Time Web Interface

SOURCE CODE

VERILOG CODE :-

module parking_top(

input  wire clk,

input  wire rx,

output wire tx,

output wire [7:0] led,

output wire servo_pwm,

output wire buzzer

);

wire [7:0] rx_data;

wire       rx_done;

wire       tx_busy;

reg [7:0]  slot_reg   = 8'b00000000;

reg [7:0]  tx_data    = 8'b00000000;

reg        tx_start   = 1'b0;

reg        servo_trig = 1'b0;

// Buzzer

reg [23:0] buzz_count  = 0;

reg        buzz_active = 1'b0;

reg [11:0] pwm_count   = 0;

reg        buzzer_pwm  = 1'b0;

// All slots full beep = 1 second

// Single slot beep    = 100ms

localparam BEEP_SHORT = 24'd2400000;   // 100ms

localparam BEEP_LONG  = 24'd48000000;  // 2 seconds at 24MHz

localparam PWM_HALF   = 12'd1250;      // 1kHz tone at 24MHz

assign led    = slot_reg;

assign buzzer = buzzer_pwm;

uart_rx #(.CLK_FREQ(24000000),.BAUD_RATE(9600)) UART_RX (

.clk(clk),.rx(rx),.rx_data(rx_data),.rx_done(rx_done)

);

uart_tx #(.CLK_FREQ(24000000),.BAUD_RATE(9600)) UART_TX (

.clk(clk),.tx_data(tx_data),.tx_start(tx_start),.tx(tx),.tx_busy(tx_busy)

);

servo_ctrl #(.CLK_FREQ(24000000)) SERVO (

.clk(clk),.trigger(servo_trig),.servo_pwm(servo_pwm)

);

// PWM tone generator for buzzer

always @(posedge clk) begin

if(buzz_active) begin

if(pwm_count >= PWM_HALF - 1) begin

pwm_count  <= 0;

buzzer_pwm <= ~buzzer_pwm;

end else

pwm_count <= pwm_count + 1;

end else begin

buzzer_pwm <= 1'b0;

pwm_count  <= 0;

end

end

// Main logic

always @(posedge clk) begin

tx_start   <= 1'b0;

servo_trig <= 1'b0;

// Buzzer countdown

if(buzz_active) begin

if(buzz_count == 0)

buzz_active <= 1'b0;

else

buzz_count <= buzz_count - 1;

end

if(rx_done) begin

case(rx_data)

8'h31: begin

slot_reg[0] <= ~slot_reg[0];

tx_data     <= {slot_reg[7:1], ~slot_reg[0]};

servo_trig  <= 1'b1;

if(!slot_reg[0]) begin buzz_active<=1'b1; buzz_count<=BEEP_SHORT; end

end

8'h32: begin

slot_reg[1] <= ~slot_reg[1];

tx_data     <= {slot_reg[7:2], ~slot_reg[1], slot_reg[0]};

if(!slot_reg[1]) begin buzz_active<=1'b1; buzz_count<=BEEP_SHORT; end

end

8'h33: begin

slot_reg[2] <= ~slot_reg[2];

tx_data     <= {slot_reg[7:3], ~slot_reg[2], slot_reg[1:0]};

if(!slot_reg[2]) begin buzz_active<=1'b1; buzz_count<=BEEP_SHORT; end

end

8'h34: begin

slot_reg[3] <= ~slot_reg[3];

tx_data     <= {slot_reg[7:4], ~slot_reg[3], slot_reg[2:0]};

if(!slot_reg[3]) begin buzz_active<=1'b1; buzz_count<=BEEP_SHORT; end

end

8'h35: begin

slot_reg[4] <= ~slot_reg[4];

tx_data     <= {slot_reg[7:5], ~slot_reg[4], slot_reg[3:0]};

if(!slot_reg[4]) begin buzz_active<=1'b1; buzz_count<=BEEP_SHORT; end

end

8'h36: begin

slot_reg[5] <= ~slot_reg[5];

tx_data     <= {slot_reg[7:6], ~slot_reg[5], slot_reg[4:0]};

if(!slot_reg[5]) begin buzz_active<=1'b1; buzz_count<=BEEP_SHORT; end

end

8'h37: begin

slot_reg[6] <= ~slot_reg[6];

tx_data     <= {slot_reg[7], ~slot_reg[6], slot_reg[5:0]};

if(!slot_reg[6]) begin buzz_active<=1'b1; buzz_count<=BEEP_SHORT; end

end

8'h38: begin

slot_reg[7] <= ~slot_reg[7];

tx_data     <= {~slot_reg[7], slot_reg[6:0]};

if(!slot_reg[7]) begin buzz_active<=1'b1; buzz_count<=BEEP_SHORT; end

end

default: tx_data <= slot_reg;

endcase

// Check if ALL slots will be occupied after this toggle

// If new slot_reg == 8'hFF -> long beep 1 second

if(({slot_reg[7:1], ~slot_reg[0]} == 8'hFF) ||

({slot_reg[7:2], ~slot_reg[1], slot_reg[0]} == 8'hFF) ||

({slot_reg[7:3], ~slot_reg[2], slot_reg[1:0]} == 8'hFF) ||

({slot_reg[7:4], ~slot_reg[3], slot_reg[2:0]} == 8'hFF) ||

({slot_reg[7:5], ~slot_reg[4], slot_reg[3:0]} == 8'hFF) ||

({slot_reg[7:6], ~slot_reg[5], slot_reg[4:0]} == 8'hFF) ||

({slot_reg[7], ~slot_reg[6], slot_reg[5:0]} == 8'hFF) ||

({~slot_reg[7], slot_reg[6:0]} == 8'hFF)) begin

buzz_active <= 1'b1;

buzz_count  <= BEEP_LONG;

end

if(!tx_busy) tx_start <= 1'b1;

end

end

endmodule