FPGA-Based Smart Parking Management System with Real-Time Web Interface

SOURCE CODE

Constraints:-

# ============================================================

# AT-STLN-ARTIX 7-001 (Anmaya Technologies)

# XDC Constraints for parking_top

# ============================================================

# Clock - 24 MHz oscillator (ASDM1) on D13

set_property PACKAGE_PIN D13 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

create_clock -period 41.667 -name sys_clk [get_ports clk]

# UART - using PMOD connector pins (Bank 34)

# TX  → PMOD IO_0 = T2

# RX  → PMOD IO_1 = R3

set_property PACKAGE_PIN T2 [get_ports tx]

set_property IOSTANDARD LVCMOS33 [get_ports tx]

set_property PACKAGE_PIN R3 [get_ports rx]

set_property IOSTANDARD LVCMOS33 [get_ports rx]

# Servo PWM - DC Motor driver IN2 pin = H11 (Bank 14)

set_property PACKAGE_PIN H11 [get_ports servo_pwm]

set_property IOSTANDARD LVCMOS33 [get_ports servo_pwm]

# Buzzer - BZR_OUT on K5 (Bank 35) - confirmed from hardware manual section 4.11

set_property PACKAGE_PIN K5 [get_ports buzzer]

set_property IOSTANDARD LVCMOS33 [get_ports buzzer]

# User LEDs - Bank 35 (section 3.1)

set_property PACKAGE_PIN D5  [get_ports {led[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[0]}]

set_property PACKAGE_PIN A3  [get_ports {led[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[1]}]

set_property PACKAGE_PIN B4  [get_ports {led[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[2]}]

set_property PACKAGE_PIN A4  [get_ports {led[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[3]}]

set_property PACKAGE_PIN E6  [get_ports {led[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[4]}]

set_property PACKAGE_PIN C13 [get_ports {led[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[5]}]

set_property PACKAGE_PIN C14 [get_ports {led[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[6]}]

set_property PACKAGE_PIN D14 [get_ports {led[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[7]}]