FPGA-Based Smart Parking Management System with Real-Time Web Interface

SOURCE CODE

FLASK CODE OF PYTHON:-

from flask import Flask, render_template

from flask_socketio import SocketIO

import serial, serial.tools.list_ports, threading, time

app = Flask(__name__)

socketio = SocketIO(app, cors_allowed_origins="*")

ser = None

def find_port():

ports = list(serial.tools.list_ports.comports())

for p in ports:

desc = p.description.lower()

if any(x in desc for x in ["usb", "uart", "serial", "cp210", "ch340", "ftdi"]):

print("Found port:", p.device, "-", p.description)

return p.device

if ports:

print("Using first port:", ports[0].device)

return ports[0].device

print("No port found!")

return None

def connect_serial():

global ser

while True:

try:

if ser is None or not ser.is_open:

port = find_port()

if port:

ser = serial.Serial(port, 9600, timeout=1)

print("Connected to", port)

socketio.emit("port_status", {"connected": True, "port": port})

else:

socketio.emit("port_status", {"connected": False, "port": ""})

time.sleep(3)

except Exception as e:

print("Serial error:", e)

ser = None

socketio.emit("port_status", {"connected": False, "port": ""})

time.sleep(3)

time.sleep(1)

def read_serial():

global ser

while True:

try:

if ser and ser.is_open and ser.in_waiting:

data = ser.read(1)

val = int.from_bytes(data, "big")

# bit0 = Slot1, bit1 = Slot2 ... bit7 = Slot8

slots = [(val >> i) & 1 for i in range(8)]

print("Received:", hex(val), "-> slots:", slots)

socketio.emit("slot_update", {"slots": slots})

except Exception as e:

print("Read error:", e)

ser = None

time.sleep(0.01)

@app.route("/")

def index():

return render_template("index.html")

@socketio.on("toggle_slot")

def toggle_slot(data):

global ser

try:

if ser and ser.is_open:

slot = data["slot"]

cmd = str(slot).encode()  # ASCII '1'-'8' matches Verilog 0x31-0x38

ser.write(cmd)

print(f"Sent slot {slot} as ASCII {hex(ord(str(slot)))}")

else:

print("Serial not connected")

except Exception as e:

print("Write error:", e)

if __name__ == "__main__":

threading.Thread(target=connect_serial, daemon=True).start()

threading.Thread(target=read_serial, daemon=True).start()

socketio.run(app, port=5000, debug=False)