FPGA-Based Smart Parking Management System with Real-Time Web Interface

SOURCE CODE

RX CODE:-

module uart_rx

#(

parameter CLK_FREQ  = 24000000,

parameter BAUD_RATE = 9600

)

(

input  clk,

input  rx,

output reg [7:0] rx_data,

output reg       rx_done

);

localparam CLKS_PER_BIT = CLK_FREQ / BAUD_RATE;

reg [15:0] clk_count;

reg [2:0]  bit_index;

reg [7:0]  rx_shift;

reg [1:0]  state;

reg rx_sync1, rx_sync2;

always @(posedge clk) begin

rx_sync1 <= rx;

rx_sync2 <= rx_sync1;

end

localparam IDLE  = 2'd0;

localparam START = 2'd1;

localparam DATA  = 2'd2;

localparam STOP  = 2'd3;

always @(posedge clk)

begin

rx_done <= 0;

case(state)

IDLE:

begin

if(rx_sync2 == 0)

begin

clk_count <= CLKS_PER_BIT/2 - 1;

state     <= START;

end

end

START:

begin

if(clk_count == 0)

begin

if(rx_sync2 == 0)

begin

clk_count <= CLKS_PER_BIT - 1;

bit_index <= 0;

state     <= DATA;

end

else

state <= IDLE;

end

else

clk_count <= clk_count - 1;

end

DATA:

begin

if(clk_count == 0)

begin

rx_shift[bit_index] <= rx_sync2;

if(bit_index == 7)

begin

clk_count <= CLKS_PER_BIT - 1;

state     <= STOP;

end

else

begin

clk_count <= CLKS_PER_BIT - 1;

bit_index <= bit_index + 1;

end

end

else

clk_count <= clk_count - 1;

end

STOP:

begin

if(clk_count == 0)

begin

if(rx_sync2 == 1)

begin

rx_data <= rx_shift;

rx_done <= 1;

end

state <= IDLE;

end

else

clk_count <= clk_count - 1;

end

default: state <= IDLE;

endcase

end

endmodule