FPGA-Based Smart Parking Management System with Real-Time Web Interface

SOURCE CODE

TX CODE:-

module uart_tx

#(

parameter CLK_FREQ  = 24000000,

parameter BAUD_RATE = 9600

)

(

input  clk,

input  [7:0] tx_data,

input        tx_start,

output reg   tx,

output reg   tx_busy

);

localparam CLKS_PER_BIT = CLK_FREQ / BAUD_RATE;

reg [15:0] clk_count;

reg [2:0]  bit_index;

reg [7:0]  tx_shift;

reg [1:0]  state;

localparam IDLE  = 2'd0;

localparam START = 2'd1;

localparam DATA  = 2'd2;

localparam STOP  = 2'd3;

initial begin

state     = IDLE;

tx        = 1'b1;

tx_busy   = 0;

clk_count = 0;

bit_index = 0;

end

always @(posedge clk)

begin

case(state)

IDLE:

begin

tx      <= 1'b1;

tx_busy <= 0;

if(tx_start)

begin

tx_shift <= tx_data;

tx_busy  <= 1;

state    <= START;

end

end

START:

begin

tx <= 1'b0;

if(clk_count == CLKS_PER_BIT - 1)

begin

clk_count <= 0;

bit_index <= 0;

state     <= DATA;

end

else

clk_count <= clk_count + 1;

end

DATA:

begin

tx <= tx_shift[bit_index];

if(clk_count == CLKS_PER_BIT - 1)

begin

clk_count <= 0;

if(bit_index == 7)

state <= STOP;

else

bit_index <= bit_index + 1;

end

else

clk_count <= clk_count + 1;

end

STOP:

begin

tx <= 1'b1;

if(clk_count == CLKS_PER_BIT - 1)

begin

clk_count <= 0;

state     <= IDLE;

end

else

clk_count <= clk_count + 1;

end

default: state <= IDLE;

endcase

end

endmodule